RiverMonitor Sensor Data API and Sitehost Deployment Notes

Prepared from the technical discussion, API requirements and sitehost investigation

| Item | Description |
| --- | --- |
| Purpose | To explain how the sensor provider should expose data via an API for the dashboard, and to document the Aber sitehost / SMB investigation. |
| Related domain | https://rivermonitor.aber.ac.uk |
| Mounted SMB share | /Volumes/rivermonitor-aber-ac-uk |
| Prepared on | 2026-04-22 |

Executive Summary

An API is a secure machine-readable endpoint that allows another system to retrieve data automatically.

The sensor provider should collect sensor readings into its own cloud database, then expose a REST API for the dashboard to poll.

The Aber sitehost SMB share has been mounted successfully, but it appears to be a static hosting space rather than the active deployment directory for rivermonitor.aber.ac.uk.

A test file placed under public_html was not served at https://rivermonitor.aber.ac.uk/test.txt, which indicates that the live site is not reading from that directory.

The live site appears to be a Next.js application. To modify the dashboard or add API routes to it, the active Next.js source code and deployment process must be located.

1. What an API Means in This Project

In plain terms, an API is a controlled data access point for another computer system. A web page is for humans to look at; an API is for software to read.

| Concept | Plain-English Meaning | Meaning in This Project |
| --- | --- | --- |
| Web page | A human-facing screen | The dashboard, login page and charts |
| API | A data access endpoint for software | The endpoint the dashboard can poll to retrieve sensor readings |
| JSON | A structured data format programs can read | Fields such as timestamp, device_id, value and units |
| API Key / Token | A digital key | Prevents public access to sensor data |

An API is not the sensor and it is not the database itself. It is usually an HTTPS URL such as:

https://api.yourcompany.com/api/v1/readings

When the dashboard calls this URL with a time range, the API returns the relevant sensor readings as JSON.

2. Recommended Data Flow

As the sensor and deployment provider, the recommended architecture is:

Sensors
  -> 4G / 5G / LoRaWAN / gateway
  -> Provider cloud ingestion service
  -> Database storage
  -> REST API
  -> Ceredigion / West Wales dashboard polling
  -> Dashboard visualisation

The key point is that the dashboard should not normally connect directly to field sensors. Sensor data should first be collected into a controlled backend system, then exposed through an API.

3. Components the Sensor Provider Needs to Build

| Component | Role | Minimum Requirement |
| --- | --- | --- |
| Sensor data uplink | Transmit readings from the field | 4G/5G, LoRaWAN, Wi-Fi or gateway-based upload |
| Cloud ingestion service | Receive uploaded sensor data | Stable writes, basic validation and retry handling |
| Database | Store time-series readings | At minimum: device_id, timestamp, value and units |
| REST API | Allow the dashboard to retrieve readings | HTTPS, authentication and from/to time-range support |
| API documentation | Explain how the integration works | Base URL, authentication, fields, examples and device ID list |

4. Minimum API Fields

| Field | Required | Example | Notes |
| --- | --- | --- | --- |
| device_id / station_id | Required | EDORE-001 | Stable unique identifier for each sensor |
| timestamp | Required | 2026-04-13T14:30:00Z | Collection time, preferably UTC ISO 8601 |
| value | Required | 21.48 | Numeric reading |
| units | Strongly recommended | µg/L-P, mg/L-N, °C, NTU | Avoids ambiguity across parameters |
| parameter | Recommended | Phosphate, Nitrate, pH | If omitted, the dashboard team may map this from deployment records |
| qc_status | Optional | accepted / rejected | Quality status or rejection reason |
| battery_voltage | Optional | 12.7 | Useful for maintenance diagnostics |
| signal_strength | Optional | -83 dBm | Useful for communications diagnostics |

5. Suggested API Design

A minimum useful read endpoint would be:

GET /api/v1/readings?from=2026-04-13T00:00:00Z&to=2026-04-14T00:00:00Z

Example JSON response:

[
  {
    "device_id": "EDORE-001",
    "timestamp": "2026-04-13T14:30:00Z",
    "parameter": "Phosphate",
    "value": 21.48,
    "units": "µg/L-P",
    "qc_status": "accepted"
  },
  {
    "device_id": "EDORE-002",
    "timestamp": "2026-04-13T14:35:00Z",
    "parameter": "Nitrate",
    "value": 1.26,
    "units": "mg/L-N",
    "qc_status": "accepted"
  }
]

Recommended query parameters:

| Parameter | Description |
| --- | --- |
| from | Start time in ISO 8601 format |
| to | End time in ISO 8601 format |
| device_id | Optional filter for one sensor |
| parameter | Optional filter by measurement type |
| limit / cursor | Pagination for larger result sets |

6. Security and Authentication Notes

The API should use HTTPS. Plain HTTP is not recommended for production.

The API should not be publicly open without authentication. Provide an API key or Bearer token to the dashboard team.

Tokens should be revocable and rotatable, and should not be embedded in public frontend code.

API request logs should be kept so polling and integration issues can be diagnosed.

If station data is sensitive, consider IP restrictions, rate limits and device-level access controls.

Authorization: Bearer YOUR_API_KEY

7. Possible Technical Approaches

| Approach | Advantages | Cautions |
| --- | --- | --- |
| Python FastAPI + PostgreSQL | Simple and fast for REST API development | Requires cloud server or container deployment |
| Node.js / Next.js API routes | Useful if the live dashboard is already a Next.js application | Requires access to the real Next.js project and deployment process |
| Supabase + Edge Functions | Low operational overhead and quick to build | Confirm data governance and external access requirements |
| Azure / AWS managed services | Enterprise-grade reliability and scaling | More configuration effort and higher cost |
| Static JSON file upload only | Very simple | Not suitable for secure long-term incremental data integration |

For this project, the most robust first version is likely an independent sensor ingestion service, database and HTTPS API that the dashboard team can poll.

8. Minimum Database Table Suggestion

| Field | Example Type | Purpose |
| --- | --- | --- |
| id | uuid / bigint | Internal primary key |
| device_id | text | Unique sensor identifier |
| timestamp | timestamptz | Reading timestamp |
| parameter | text | Measurement type |
| value | numeric / double | Reading value |
| units | text | Measurement units |
| qc_status | text | Quality status |
| battery_voltage | numeric | Optional battery diagnostic |
| raw_payload | jsonb | Original incoming payload for traceability |
| created_at | timestamptz | Database insertion time |

9. Aber Sitehost Mounting and Investigation Findings

The Aber sitehost SMB share was successfully mounted on macOS. The mounted path is:

/Volumes/rivermonitor-aber-ac-uk

The working SMB path was:

smb://sitehost.disk.aber.ac.uk/rivermonitor-aber-ac-uk

The mounted share contains mainly:

/Volumes/rivermonitor-aber-ac-uk
  public_html/
  index.html
  package.json
  package-lock.json
  EMWAC/MeetingNotes
  black covert 3.5.24 to 28.6.24.csv

The package.json file contains only:

{}

No typical Next.js project files or folders were found in this mounted share, such as .next, pages, app, src, next.config.js or server.js.

10. Why public_html Does Not Appear to Be Active

A test file was placed under public_html, but the following URL returned a 404 page:

https://rivermonitor.aber.ac.uk/test.txt

The 404 page appears to be generated by the live Next.js application. This indicates that:

The sitehost SMB share exists and is writable.

The current live rivermonitor.aber.ac.uk site is not serving files directly from this public_html directory.

The live domain is likely being handled by nginx or another reverse proxy pointing to a separate Next.js application, VM or container.

Therefore, placing API files or static assets in public_html will not make them available to the current dashboard.

In practical terms, access has been granted to a static hosting area, but the currently live application is running somewhere else.

11. Access Needed to Modify the Existing Dashboard

To add API routes to the existing dashboard or modify the live site, one of the following access paths is required:

Access to the actual Git repository containing the live Next.js application.

Access to the VM, container or deployment platform running the Next.js app.

Documentation for the nginx / reverse proxy configuration for rivermonitor.aber.ac.uk.

The deployment process, such as npm run build, pm2 restart, Docker deployment or CI/CD workflow.

The database connection and environment variable management process.

If this access is not available, it is not advisable to try to build the API inside the Aber sitehost share. A separate sensor API service is more realistic.

12. Suggested Questions for Aber IT / Project Owner

The following message can be sent directly:

Hi,

I have access to sitehost-disk-rivermonitor-aber-ac-uk and can mount it on macOS via SMB. I can see the public_html folder, but files placed there, for example test.txt, are not served at https://rivermonitor.aber.ac.uk/test.txt.

The live site appears to be a Next.js application behind nginx rather than a static site served from public_html.

Could you please confirm:
1. Is sitehost-disk-rivermonitor-aber-ac-uk still connected to the live rivermonitor.aber.ac.uk domain?
2. Where is the active Next.js application hosted? Is it on a VM, container, or separate deployment platform?
3. Where is the source code repository for the live application?
4. What is the correct deployment process if we need to add API routes or update the dashboard?
5. If we deploy our own sensor API separately, can the dashboard consume data from an external HTTPS REST API?

Many thanks.

13. Recommended Next Steps

| Priority | Action | Purpose |
| --- | --- | --- |
| High | Confirm the real Next.js project location and deployment access | Determine whether API routes can be added to the existing dashboard |
| High | Design the provider-side sensor data API | Create a stable data source regardless of dashboard access |
| High | Prepare the device ID list and field format | Allow the dashboard team to map sensors correctly |
| Medium | Choose the field communications method: 4G, LoRaWAN or gateway | Ensure readings are transmitted reliably |
| Medium | Choose the cloud platform and database | Store historical readings and support querying |
| Medium | Prepare API documentation and a test token | Allow the dashboard team to start integration testing |

14. Final Assessment

The Aber sitehost share can be mounted and written to, but it does not appear to be the active deployment target for the live rivermonitor.aber.ac.uk site. The live site is a Next.js application, and the actual source code and deployment location must be identified before the existing dashboard can be modified or API routes can be added to it.

For the sensor provider, the safest technical path is to build an independent sensor ingestion, storage and API service, then provide the dashboard team with the API base URL, authentication method, field definitions, example responses and device ID list.

Prepared from the 2026-04-22 technical discussion, Ceredigion API requirements, Aber sitehost/SMB investigation and live site access tests.